Physics 3310

Homework

Practice Problems

With

Rotational Motion

Chapter 8

Giancoli

_________________________

STUDENT NAME

_________________________

TEACHER’S NAME

______________

PERIOD

Questions

- A bicycle odometer (which measures distance traveled) is attached near the wheel hub and is designed for 27-inch wheels. What happens if you use it on a bicycle with 24-inch wheels?

- Suppose a record turntable rotates at constant angular velocity. Does a point on the rim have radial and/or tangential acceleration? If the turntable’s angular velocity increases uniformly, does the point have radial and/or tangential acceleration? For which cases would the magnitude of either of the components of linear acceleration change?

- Can a small force exert a greater torque than a larger force? Explain

- If a force F acts on a body such that its lever arm is zero, does it have any effect on the body’s motion?

- Why is it more difficult to do a sit-up with your hands behind your head than when they are out-stretched in front of you? A diagram may help you answer this.

- A 21-speed bicycle has seven sprockets at the rear wheel and three at the pedal cranks. In which gear is it harder to pedal, a small rear sprocket or a large rear sprocket? Why? In which gear is it harder to pedal, a small front sprocket or a large front sprocket? Why?

- Mammals that depend on being able to run fast have slender lower legs with flesh and muscle concentrated high, close to the body (Fig. 8-33). On the basis of rotational dynamics, explain why this distribution of mass is advantageous.

- Why do tightrope walkers carry a long, narrow beam (Fig. 8-34)?

- If the net force on a system is zero, is the net torque also zero? If the net torque on a system is zero, is the net force zero?

- Two inclines have the same height but make different angles with the horizontal. The same steel ball is rolled down each incline. On which incline will the speed of the ball at the bottom be greatest? Explain.

- Two solid spheres simultaneously start rolling (from rest) down an incline. One sphere has twice the radius and twice the mass of the other. Which reaches the bottom of the incline first? Which has the greater speed there? Which has the greater speed there? Which has the greater total kinetic energy at the bottom?

- A sphere and a cylinder have the same radius and the same mass. They start from rest at the top of an incline. Which reaches the bottom first? Which has greater total kinetic energy at the bottom? Which has the greater rotational KE?

- A cyclist rides over the top of a hill. Is the bicycle’s motion rotational, translational, or a combination of both?

- We claim that momentum and angular momentum are conserved. Yet most moving or rotating bodies eventually slow down and stop. Explain

- If there were a great migration of people toward the equator, how would this affect the length of the day?

- In what direction is the Earth’s angular velocity for its daily rotation on it axis?

- When a motorcyclist leaves the ground on a jump, if the throttle is left on (so that the rear wheel spins), why does the front of the cycle rise up?

- Suppose you are standing on the edge of a large rotating turntable. What happens if you walk towards the center?

- A quarterback leaps into the air to throw a forward pass. As he throws the ball, the upper part of his body rotates. If you look quickly you will notice that his hips and legs rotate in the opposite direction (Fig. 8-35). Explain.

- Look at the face of a clock with the second hand. In what direction is the angular momentum of the second hand?

- On the basis of the law of conservation of angular momentum, discuss why a helicopter must have more than one rotor (or propeller). Discuss one or more ways the second propeller can operate in order to keep the body stable.

Problems

- (I) A laser beam is directed at the Moon, 380,000 km from Earth. The beam diverges at and angle  (Fig. 8-36) of 1.8 x 10-5 rad. What diameter spot will it make on the Moon?

- (I) A 0.35-m diameter grinding wheel rotates at 1800 rpm. Calculate its angular velocity in rad/s.

- (I) A 33-rpm phonograph record reaches its rated speed 1.8s after it is turned on. What was the angular acceleration?

- (II) A child rolls a ball on a level floor 4.5 m to another child. If the ball makes 15.0 revolutions, what is its diameter?

- (II) A bicycle with 68-cm-diameter tires travels 7.0 km. How many revolutions do the wheels make?

- (I) A centrifuge accelerates from rest to 15,000 rpm in 220s. Through how many revolutions did it turn in this time?

- (I) An automobile engine slows down from 4000 rpm to 1200 rpm in 3.5s. Calculate (a) its angular acceleration, assumed uniform, and (b) the total number of revolutions the engine makes in this time.

- (II) Pilots can be tested for the stresses of flying high-speed jets in a whirling “human centrifuge” which takes 1.0 min to turn 20 complete revolutions before reaching its final speed. (a) What was its angular acceleration (assume constant), and (b) what was its final speed in rpm?

- (II) A small rubber wheel is used to drive a large pottery wheel, and they are mounted so that their circular edges touch. If the small wheel has a radius of 2.0 cm and accelerates at the rate of 7.2 rad/s2, and it is in contact with the pottery wheel (radius 25.0 cm) without slipping, calculate (a) the angular acceleration of the pottery wheel, and (b) the time it takes the pottery wheel to reach its required speed of 65 rpm.

- (II) The tires of a car make 65 revolutions as the car reduces its speed uniformly from 100 km/h to 50 km/h. The tires have a diameter of 0.80 m. (a) What was the angular acceleration? (b) If the car continues to decelerate at this rate, how much more time is required for it to stop?

- (I) What is the maximum torque exerted by a 55-kg person riding a bike if the rider puts all her weight on each pedal when climbing a hill? The pedals rotate in a circle of radius 17 cm.

- (I) A person exerts a force of 45 N on the end of a door 84 cm wide. What is the magnitude of the torque if the force is exerted (a) perpendicular top the door, and (b) at a 60 angle to the face of the door?

- (II) Calculate the net torque about the axle of the wheel shown in Fig. 8-37. Assume that a friction torque of 0.40 mN opposes the motion.

- (I) Calculate the moment of inertia of a 12.2-kg sphere of radius 0.623 m when the axis of rotation is through its center.

- (II) Calculate the moment of inertia of the array of point objects shown in Fig. 8-39 about (a) the vertical axis, and (b) the horizontal axis. Assume the objects are wired together by very light rigid pieces of wire. About which axis would be harder to accelerate this array? In Fig.8-39, m = 1.8 kg and M = 3.1 kg. The array is rectangular and it is split through the middle by the horizontal axis.

- (II) In order to get a flat uniform cylindrical satellite spinning at the correct rate, engineers fire the four tangential rockets as shown in Fig. 8-40. If the satellite has a mass of 2600 kg and a radius of 3.0 m, what it the required steady force of each rocket if the satellite is to reach 30 rpm in 5.0 min?

- (II) A grinding wheel is a uniform cylinder with a radius of 8.50 cm and a mass of 0.580 kg. Calculate (a) its moment of inertia about its center, and (b) the applied torque needed to accelerate it from rest to 1500 rpm in 5.00 s if it is known to slow down from 1500 rpm to rest in 55.0 s.

- (II) A softball player swings a bat, accelerating it from rest to 3.0 rev/s in a time of 0.20s. Approximate the bat as a 2.2-kg uniform rod of length 0.95 m, and compute the torque the player applies to one end of it.

- (II) A day-care worker pushes tangentially on a small hand-driven merry-go-round and is able to accelerate it from rest to a spinning rate of 20 rpm in 10.0s. Assume the merry-go-round is a disk of radius 2.5 m and has a mass of 800 kg, and two children (each with a mass of 25 kg) sit opposite each other on the edge. Calculate the torque required to produce the acceleration, neglecting the frictional torque. What force is required?

- (II) The radius of the roll of paper shown in Fig. 8-43 is 7.6 cm and its moment of inertia is I = 2.9 x 10-3 kgm2. A force of 3.2 N is exerted on the end of the roll for 1.3 s, but the paper does not tear so it begins to unroll. A constant friction torque of 0.11mN is exerted on the roll which gradually brings it to a stop. Assuming that the paper’s thickness is negligible, calculate (a) the length of the paper that unrolls during the time that the force is applied (1.3s), and (b) the length of paper that unrolls from the time the force ends to the time when the roll has stopped moving.

- (I) A bowling ball of mass 7.3 kg and radius 9.0 cm rolls without slipping down a lane at 4.3 m/s. Calculate its total kinetic energy.

- (I) A centrifuge rotor has a moment of inertia of 3.15 x 10-2 kgm2. How much energy is required to bring it from rest to 8000 rpm?

- (II) A merry-go-round has a mass of 1640 kg and a radius of 8.20 m. How much work is required to accelerate it from rest to a rotation rate of one revolution in 8.00s? (Assume it is a solid cylinder).

- (II) (a) Calculate the translational and rotational speeds of a sphere (radius 20.0 cm and mass 1.20 kg) that rolls without slipping down a 30º incline that is 10.0 m long, when it reaches the bottom. Assume it started from rest. (b) What is its ratio of translational to rotational KE at the bottom? Try to avoid putting in numbers until the end so you can answer: (c) do your answers in (a) and (b) depend on the radius of the sphere or its mass?

- (I) What is the angular momentum of a 0.210-kg ball rotating on the end of a string in a circle of radius 1.10m at an angular speed of 10.4 rad/s?

- (I) A person stands, hands at the side, on a platform that is rotating at a rate of 1.30 rev/s. If the person now raises his arms to a horizontal position, Fig. 8-45, the speed of rotation decreases to 0.80 rev/s. (a) Why does this occur? (b) By what factor has the moment of inertia of the person changed?

- (I) A figure skater during her finale can increase her rotation rate from an initial rate of 1.0 rev every 2.0 s to a final rate of 3.0 rev/s. If her initial moment of inertia was 4.6 kg·m2, what is her final moment of inertia? How does she physically accomplish this change?

- A person of mass 75 kg stands at the center of a rotating merry-go-round platform of radius 3.0 m and moment of inertia 1000 kg·m2. The platform rotates without friction with angular velocity 2.0 rad/s. The person walks radially to the edge of the platform. (a) Calculate the angular velocity when the person reaches the edge. (b) Compare the rotational kinetic energies of the system of platform plus person before and after the person’s walk.